Model and specification of combined orthopedic external fixation bracket

| model | Part name | Part model | material quality | Component specifications（mm） | number |
| --- | --- | --- | --- | --- | --- |
| WJZⅠ-01A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ3.0~Φ5.5, interval0.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T | (Φ5.0、Φ5.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ2.0、Φ2.5、3.0、3.5）/(Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0） | 2 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅠ-02A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ3.0~Φ5.5,interval0.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T | (Φ5.0、Φ5.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ2.0、Φ2.5、3.0、3.5）/(Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0） | 2 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0~Φ6.0,interval0.5）/(Φ3.0~Φ6.0,interval0.5） | 1 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
| WJZⅠ-03A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ3.0~Φ5.5,interval0.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T | (Φ5.0、Φ5.5）×（50～200，interval5）、Φ6.0×（80～300，interval5） |  |
|  | Nail bar clamp | WJZJ02A(B) | Sc/T | (Φ2.0、Φ2.5、3.0、3.5）/(Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0） | 2 |
| WJZⅠ-04A(B) | Joint connector | WJZG01/02/03 A(B) | Sc/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/（50～250，interval5） | 1 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ2.0、Φ2.5、Φ3.0）/(Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0） (Φ3.5）/(Φ3.5、Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅠ-05A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0、Φ6.0）×（50～200，interval5）、Φ8.0×（100～200，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |

|  |  | WJZK01/02A(B) | Sc/T |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ5.0/Φ5.0、Φ6.0/Φ6.0、Φ8.0/Φ8.0 | 4 |
|  | Rod clamp | WJZB02/03A(B) | Sc/T | Φ5.0/Φ5.0、Φ6.0/Φ6.0、Φ8.0/Φ8.0 | 4 |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ2.0、Φ2.5、Φ3.0、Φ3.5、Φ4.0）/(Φ5.0、Φ6.0、Φ8.0） | 2 |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ5.0、Φ6.0、Φ8.0）×（50～200，interval5） | 4 |
| WJZⅠ-06A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEKT | (Φ5.0、Φ6.0）×（50～200，interval5）、Φ8.0×（100～200，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ5.0/Φ5.0、Φ6.0/Φ6.0、Φ8.0/Φ8.0 | 1 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0、Φ6.0、Φ8.0） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-01A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 1 |
|  | Clip block | WJZD02A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 1 |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（50～200，interval5） | 1 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ5.0~Φ8.0，interval0.5）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
| WJZⅡ-02A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |

| WJZⅡ-03A(B) | Connecting rod | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 2 |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Connecting pipe | WJZK01/02A(B) | Sc/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 2 |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ5.0~Φ8.0，interval0.5）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（50～200，interval5） | 2 |
| WJZⅡ-04A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ5.0~Φ8.0，interval0.5）/(Φ5.0~Φ8.0，interval0.5） | 4 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（50～200，interval5） | 4 |
| WJZⅡ-05A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS05A(B) | Sc/AL/Cf/PEEK/T | (Φ5.0~Φ8.0，interval0.5）×（80～300，interval5） | 1 |
|  |  | WJZK05A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0）/(Φ5.0~Φ8.0，interval0.5） | 3 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ5.0~Φ8.0，interval0.5）/(Φ5.0~Φ8.0，interval0.5） | 2 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |

| WJZⅡ-06A(B) | Connecting rod | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 3 |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Connecting pipe | WJZK01/02A(B) | Sc/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 3 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 3 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 5 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-07A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 2 |
|  |  | WJZB02/03(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（50～200，interval5） | 2 |
| WJZⅡ-08A(B) | Joint connector | WJZG01/02/03 A(B) | Sc/T | (Φ6.5、Φ7.0、Φ7.5、Φ8.0）/（150～400，interval5） | 1 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ6.5、Φ7.0、Φ7.5、 Φ8.0） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-09A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-10A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |

4

|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 2 |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |
|  | pillar | WJZZ02A(B) | Sc/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（50～200，interval5） | 2 |
| WJZⅡ-11A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Connecting rod / pipe | WJZK01/02A(B) | Sc/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 6 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 2 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
| WJZⅡ-12A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 4 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 1 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
| WJZⅡ-13A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 4 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |

5

|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 1 |
| --- | --- | --- | --- | --- | --- |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（50～200，interval5） | 2 |
| WJZⅡ-14A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（140～300，interval5） | 1 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 3 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 4 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 1 |
|  | pillar | WJZZ01/02A(B) | Sc/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（50～200，interval5） | 2 |
| WJZⅡ-15A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（140～300，interval5） | 1 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS04A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（140～300，interval5） | 1 |
|  |  | WJZK04A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 3 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 5 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-16 | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 1 |

| A(B) |  | WJZS02A(B) | Sc/AL/T |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（140～300，interval5） | 2 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Adjusting lever | WJZT01A(B) | Sc/Sc+AL/T | (Φ7.0、Φ7.5、Φ8.0）×（200～400，interval5） | 2 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 6 |
|  |  | WJZB02/03A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 6 |
|  |  | WJZJ03/05A(B) | Sc/T |  |  |
| WJZⅡ-17A(B) | Adjusting lever | WJZT02A(B) | Sc/T | (Φ7.0、Φ7.5、Φ8.0）×（200～400，interval5） | 1 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
| WJZⅡ-18A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ02A(B) | Sc/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |
| WJZⅡ-19A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～200，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 1 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 1 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 1 |
|  |  | WJZB02A(B) | Sc/T |  |  |
| WJZⅡ-20 | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ7.0、Φ7.5、Φ8.0）×（80～200，interval5） | 3 |

| A(B) |  | WJZS02A(B) | Sc/AL/T |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WJZK01/02A(B) | Sc/T |  |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ7.0/Φ7.0、Φ7.5/Φ7.5、Φ8.0/Φ8.0 | 2 |  |
|  |  | WJZB02/03 A(B) | Sc/T |  |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ7.0、Φ7.5、Φ8.0） | 2 |  |
|  |  | WJZJ03/05 A(B) | Sc/T |  |  |  |
|  | Nail bar clamp | WJZJ04A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ3.0、Φ3.5、Φ4.0、Φ4.5、Φ5.0）/(Φ5.0、Φ5.5、Φ6.0、 Φ6.5、Φ7.0、Φ7.5、Φ8.0） | 1 |  |
| WJZⅡ-21A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |  |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 2 |  |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |  |
|  | Standard Taylor rod | WJZW01A(B) | Sc/AL/T | L50～L300，interval5 | 1 |  |
|  | Quick mount Taylor bar | WJZW02A(B) | Sc/AL/T | L50～L300，interval5 | 2 |  |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval5 | 1 |  |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval5 | 4 |  |
|  | Offset slot needle base | WJZL07A(B) | Sc/AL/T | L10～L150，interval5 | 1 |  |
|  | Universal joint | WJZL08A(B) | Sc/AL/T | L20，L25，L30，L35，L40，L45，L50 | 6 |  |
|  | Porous connecting piece | WJZL13A(B) | Sc/AL/T | L20～L200，interval1 | 2 |  |
|  | Arc connecting piece | WJZL14A(B) | Sc/AL/T | L50～L350，interval5 | 2 |  |
|  | Threaded column | WJZL18A(B) | Sc/AL/T | L10～L150，interval5 | 1 |  |
|  | Nut column | WJZL17A(B) | Sc/AL/T | L10～L150，interval5 | 1 |  |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval5 | 2 |  |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval5 | 2 |  |
|  | bushing | WJZL22A(B) | AL/T | L10，L15，L20，L25，L30，L35，L40，L45，L50 | 6 |  |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L15，L20，L25，L30，L35，L40 | 2 |  |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval5 | 25 |  |

|  | Washer | WJZL25A(B) |  | Sc/AL/T | L5～L20,interval1 | 46 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Nut | WJZL26A(B) |  | Sc/AL/T | L5，L10，L15，L20 | 46 |
|  | earring | WJZL30A(B) |  | Sc/AL/T | L10～L100，interval5 | 5 |
|  | Ball bowl gasket | WJZL40A(B) |  | Sc/AL/T | L5，L10，L15，L20 | 1 |
|  | Marking card | WJZL41A(B) |  | AL/T | L=10 | 3 |
|  | Connecting sleeve | WJZL42A(B) |  | Sc/AL/T | L10～L100，interval5 | 1 |
| WJZⅡ-22A(B) | Six lug full ring | WJZH01A(B) |  | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | 3-lug 2/3-ring | WJZH02A(B) |  | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | General ring | WJZH06A(B) |  | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) |  | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/3 ring | WJZH08A(B) |  | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) |  | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Bending bar | WJZL02A(B) |  | Sc/AL/T | L50～L350，interval5 | 1 |
|  | Diagonal brace | WJZL03A(B) |  | Sc/AL/T | L50～L350，interval5 | 1 |
|  | Threaded rod | WJZL04A(B) |  | Sc/AL/T | L50～L350，interval5 | 11 |
|  | Hole slot needle base | WJZL05A(B) |  | Sc/AL/T | L10～L100，interval5 | 4 |
|  | Half needle base | WJZL06A(B) |  | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Unidirectional joint | WJZL09A(B) |  | Sc/AL/T | L20，L25，L30，L35，L40，L45，L50 | 2 |
|  | 90 ° conversion block | WJZL11A(B) |  | Sc/AL/T | L10～L100，interval5 | 1 |
|  | 90 ° conversion plate | WJZL12A(B) |  | Sc/AL/T | L10～L100，interval5 | 1 |
|  | 90 ° connecting piece | WJZL15A(B) |  | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Pitted nut | WJZL20A(B) |  | Sc/AL/T | L10，L15，L20，L25，L30，L35，L40，L45，L50 | 2 |
|  | Hexagonal column |  | WJZL21A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | bushing |  | WJZL22A(B) | AL/T | L10，L15，L20，L25，L30，L35，L40，L45，L50 | 6 |
|  | bolt |  | WJZL24A(B) | Sc/AL/T | L10～L100，interval5 | 4 |

|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 44 |
| --- | --- | --- | --- | --- | --- |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5，L10，L15，L20 | 44 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval5 | 1 |
|  | Indenter | WJZL28A(B) | Sc/AL/T | L15，L20，L25，L30，L35，L40，L45，L50 | 1 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval5 | 1 |
|  | Quadrangular curved column | WJZL31A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Adjustable universal hinge | WJZL32A(B) | Sc/AL/T | L10～L200，interval5 | 1 |
|  | Conversion chip | WJZL33A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Single hole connecting piece | WJZL34A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Kirschner wire guide block | WJZL35A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Steering needle base | WJZL37A(B) | Sc/AL/T | L10～L200，interval5 | 1 |
|  | Universal rail connecting piece | WJZL38A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Ring needle connecting rod | WJZL39A(B) | Sc/AL/T | L10～L300，interval5 | 1 |
|  | Ball bowl gasket | WJZL40A(B) | Sc/AL/T | L5，L10，L15，L20 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
| WJZⅡ-23A(B) | Four lug half ring | WJZH03A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Unidirectional extender | WJZL01A(B) | Sc/AL/T | L50～L300，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Universal joint | WJZL08A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L5～L400，interval2 | 1 |

|  | screw threadUnidirectional joint | WJZL10A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | Porous connecting piece | WJZL13A(B) | Sc/AL/T | L20～L200，interval1 | 1 |
|  | Track connecting piece | WJZL16A(B) | Sc/AL/T | L20～L100，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Pitted bolt | WJZL20A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 27 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 49 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 49 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval2 | 1 |
|  | earring | WJZL30A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Connecting piece | WJZL36A(B) | Sc/AL/T | L40～L200，interval2 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| WJZⅡ-24A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/3 ring | WJZH08A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Unidirectional extender | WJZL01A(B) | Sc/AL/T | L50～L300，interval2 | 1 |
|  | Bending bar | WJZL02A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | 90 ° conversion block | WJZL11A(B) | Sc/AL/T | L10～L100，interval2 | 1 |

|  | 90 ° conversion plate | WJZL12A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | 90 ° connecting piece | WJZL15A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Track connecting piece | WJZL16A(B) | Sc/AL/T | L20～L100，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 2 |
|  | Pitted nut | WJZL20A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 35 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 50 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 50 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval2 | 1 |
|  | Indenter | WJZL28A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval2 | 1 |
|  | Quadrangular curved column | WJZL31A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Adjustable universal hinge | WJZL32A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Conversion chip | WJZL33A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Single hole connecting piece | WJZL34A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Steering needle base | WJZL37A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Universal rail connecting piece | WJZL38A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| WJZⅡ-25A(B) | 3-lug 2/3-ring | WJZH02A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Four lug half ring | WJZH03A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Standard Taylor rod | WJZW01A(B) | Sc/AL/T | L50～L300，interval5 | 1 |

|  | Quick mount Taylor bar | WJZW02A(B) | Sc/AL/T | L50～L300，interval5 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | Diagonal brace | WJZL03A(B) | Sc/AL/T | L50～L350，interval5 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval5 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval5 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Offset slot needle base | WJZL07A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Universal joint | WJZL08A(B) | Sc/AL/T | L20，L25，L30，L35，L40，L45，L50 | 6 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L20，L25，L30，L35，L40，L45，L50 | 2 |
|  | screw thread Unidirectional joint | WJZL10A(B) | Sc/AL/T | L20，L25，L30，L35，L40，L45，L50 | 1 |
|  | Porous connecting piece | WJZL13A(B) | Sc/AL/T | L20～L200，interval1 | 1 |
|  | Arc connecting piece | WJZL14A(B) | Sc/AL/T | L50～L350，interval5 | 2 |
|  | Threaded column | WJZL18A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Nut column | WJZL17A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval5 | 2 |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L10，L15，L20，L25，L30，L35，L40，L45，L50 | 6 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L15，L20，L25，L30，L35，L40 | 2 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval5 | 4 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 21 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5，L10，L15，L20 | 21 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval5 | 1 |
|  | earring | WJZL30A(B) | Sc/AL/T | L10～L100，interval5 | 3 |
|  | Ring needle connecting rod | WJZL39A(B) | Sc/AL/T | L10～L300，interval5 | 1 |
|  | Ball bowl gasket | WJZL40A(B) | Sc/AL/T | L5，L10，L15，L20 | 1 |
|  | Marking card | WJZL41A(B) | AL/T | L=10 | 3 |

|  | Kirschner wire guide block | WJZL35A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
| WJZⅡ-26A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/3 ring | WJZH08A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval5 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval5 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
|  | Threaded column | WJZL18A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Nut column | WJZL17A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval5 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L10，L15，L20，L25，L30，L35，L40，L45，L50 | 6 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L15，L20，L25，L30，L35，L40 | 2 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval5 | 23 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 44 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5，L10，L15，L20 | 44 |
|  | Connecting piece | WJZL36A(B) | Sc/AL/T | L40～L200，interval5 | 1 |
|  | Ball bowl gasket | WJZL40A(B) | Sc/AL/T | L5，L10，L15，L20 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval5 | 1 |
| WJZⅢ-01A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～300，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、Φ9.5/Φ9.5、Φ10.0/Φ | 2 |

|  |  | WJZB02/03 A(B) | Sc/T | 10.0、Φ11.0/Φ11.0 |  |
| --- | --- | --- | --- | --- | --- |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 3 |
|  |  | WJZJ03/05 A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ04A(B) | Sc/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ5.0、Φ5.5、Φ6.0、 Φ6.5、Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0） | 1 |
| WJZⅢ-02A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 2 |
|  |  | WJZB02/03 A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 8 |
|  |  | WJZJ03/05 A(B) | Sc/T |  |  |
| WJZⅢ-03A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 1 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS04A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 1 |
|  |  | WJZK04A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 3 |
|  |  | WJZB02/03 A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 8 |
|  |  | WJZJ03/05 A(B) | Sc/T |  |  |
| WJZⅢ-04A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |

|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 1 |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 5 |
|  |  | WJZB02/03 A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 1 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 2 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
| WJZⅢ-05A(B) | Connecting rod | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Connecting pipe | WJZK01/02 A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、Φ9.5/Φ9.5、Φ10/Φ10、Φ11/Φ11 | 2 |
|  |  | WJZB02/03 A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8、Φ8.5、Φ9、Φ9.5、Φ10、Φ11） | 1 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11）×（50～200，interval5） | 2 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8、Φ8.5、Φ9、Φ9.5、Φ10、Φ11） | 2 |
| WJZⅢ-06A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 4 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 6 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 2 |

|  | pillar | WJZZ01/02 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 2 |
| --- | --- | --- | --- | --- | --- |
| WJZⅢ-07A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～200，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS04A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（200～400，interval5） | 2 |
|  |  | WJZK04A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 6 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
|  | Adjusting lever | WJZT01A(B) | Sc+AL/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（200～400，interval5） | 1 |
| WJZⅢ-08A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～250，interval5） | 6 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 6 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
| WJZⅢ-09A(B) | Connecting rod / pipe | WJZK01/02 A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～250，interval5） | 4 |
|  |  | WJZS01A(B) | Sc/AL/Cf/PEEK/T |  |  |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 4 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 | 4 |

|  |  | WJZJ03A(B) | Sc/T | Φ9.5、Φ10.0、Φ11.0） |  |
| --- | --- | --- | --- | --- | --- |
| WJZⅢ-10A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 3 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 6 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 2 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 4 |
| WJZⅢ-11A(B) | Connecting rod | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  | Connecting pipe | WJZK01/02 A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 1 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 1 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 2 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 4 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
| WJZⅢ-12A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） |  |

|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
| --- | --- | --- | --- | --- | --- |
|  |  | WJZJ03A(B) | Sc/T |  |  |
| WJZⅢ-13A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 4 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 2 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 2 |
| WJZⅢ-14A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 1 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS04A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～350，interval5） | 1 |
|  |  | WJZK04A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB02A(B) | Sc/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 3 |
|  | Nail bar clamp | WJZJ02/03 A(B) | Sc/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 7 |
| WJZⅢ-15A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 2 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |

|  | Adjusting lever | WJZT01A(B) | Sc+AL/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（200～400，interval5） | 2 |
| --- | --- | --- | --- | --- | --- |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 6 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 6 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
| WJZⅢ-16A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 2 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | S/T |  |  |
|  | Nail bar clamp | WJZJ02A(B) | Sc/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
| WJZⅢ-17A(B) | Adjusting lever | WJZT02A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（200～400，interval5） | 1 |
|  | Nail bar clamp | WJZJ03A(B) | Sc/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 2 |
| WJZⅢ-18A(B) | Connecting rod | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～200，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/ T |  |  |
|  | Connecting pipe | WJZK01/02 A(B) | Sc/T | Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～200，interval5） |  |
|  | Clip block | WJZD02A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 1 |
|  | Nail bar clamp | WJZJ03A(B) | Sc/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 1 |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 1 |
|  |  | WJZB02A(B) | Sc/T |  |  |
| WJZⅢ-19A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～300，interval5） | 1 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Joint connector | WJZG04A(B) | Sc/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（200～700，间 | 1 |

|  |  |  |  | 隔5） |  |
| --- | --- | --- | --- | --- | --- |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 1 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 4 |
|  |  | WJZJ02/03 A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 2 |
|  |  | WJZB02A(B) | Sc/T |  |  |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 1 |
| WJZⅢ-20A(B) | Connecting rod / pipe | WJZS01A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（80～400，interval5） | 4 |
|  |  | WJZS02A(B) | Sc/AL/T |  |  |
|  |  | WJZK01/02 A(B) | Sc/T |  |  |
|  | Connecting rod / pipe | WJZS03/05/06 A(B) | Sc/AL/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（150～300，interval5） | 2 |
|  |  | WJZK03/05/06 A(B) | Sc/T |  |  |
|  | Clip block | WJZD01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 2 |
|  | pillar | WJZZ01/02 A(B) | Sc/Cf/PEEK/T | (Φ8.0、Φ8.5、Φ9.0、Φ9.5、Φ10.0、Φ11.0）×（50～200，interval5） | 4 |
|  | Nail bar clamp | WJZJ01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | (Φ4.0、Φ4.5、Φ5.0、Φ5.5、Φ6.0）/(Φ8.0、Φ8.5、Φ9.0、 Φ9.5、Φ10.0、Φ11.0） | 3 |
|  |  | WJZJ03A(B) | Sc/T |  |  |
|  | Rod clamp | WJZB01A(B) | (Sc+AL)/(Sc+Cf)/(PEEK+Sc)/T | Φ8.0/Φ8.0、Φ8.5/Φ8.5、Φ9.0/Φ9.0、 Φ9.5/Φ9.5、Φ10.0/Φ10.0、Φ11.0/Φ11.0 | 10 |
|  |  | WJZB02 A(B) | Sc/T |  |  |
| WJZⅢ-21A(B) | U-shaped foot ring | WJZH04/05A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary U-shaped foot ring | WJZH09/10A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/3 ring | WJZH08A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |

|  | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK T | R40～R200，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 43 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 43 |
|  | Porous connecting piece | WJZL13A(B) | Sc/AL/T | L20～L200，interval1 | 2 |
|  | Connecting piece | WJZL36A(B) | Sc/AL/T | L40～L200，interval2 | 1 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | screw threadUnidirectional joint | WJZL10A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | Track connecting piece | WJZL16A(B) | Sc/AL/T | L20～L100，interval2 | 1 |
|  | Indenter | WJZL28A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval2 | 1 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval2 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| WJZⅢ-22A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 2 |
|  | Four lug half ring | WJZH03A(B) | Sc/AL/Cf/PEEKT | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Standard Taylor rod | WJZW01A(B) | Sc/AL/T | L50～L260，interval2 | 6 |

|  | Quick mount Taylor bar | WJZW02A(B) | Sc/AL/T | L50～L260，interval2 | 4 |
| --- | --- | --- | --- | --- | --- |
|  | Offset slot needle base | WJZL07A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 2 |
|  | Threaded column | WJZL18A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Bolt column | WJZL17A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 46 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 46 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 46 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | Universal joint | WJZL08A(B) | Sc/AL/T | L5～L400，interval2 | 20 |
|  | Marking card | WJZL41A(B) | AL/T | L5-L400，interval2 | 10 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| WJZⅢ-23A(B) | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary U-shaped foot ring | WJZH09/10A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | 3-lug 2/3-ring | WJZH02A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Unidirectional extender | WJZL01A(B) | Sc/AL/T | L50～L300，interval2 | 1 |
|  | Bending bar | WJZL02A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Diagonal brace | WJZL03A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval2 | 1 |

|  | Universal joint | WJZL08A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | 90 ° conversion block | WJZL11A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | 90 ° conversion plate | WJZL12A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Arc connecting piece | WJZL14A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | 90 ° connecting piece | WJZL15A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Pitted nut | WJZL20A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 48 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 48 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 48 |
|  | earring | WJZL30A(B) | Sc/AL/T | L10～L100，interval2 | 3 |
|  | Quadrangular curved column | WJZL31A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Adjustable universal hinge | WJZL32A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Conversion chip | WJZL33A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Single hole connecting piece | WJZL34A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Kirschner wire guide block | WJZL35A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Steering needle base | WJZL37A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Universal rail connecting piece | WJZL38A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Ring needle connecting rod | WJZL39A(B) | Sc/AL/T | L10～L300，interval2 | 1 |
|  | Ball bowl gasket | WJZL40A(B) | Sc/AL/T | L5-L400，interval2 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| WJZⅢ-24A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | U-SHAPED FOOT RING | WJZH04/05A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |

|  | Ordinary 1/3 ring | WJZH08A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Ordinary U-shaped foot ring | WJZH09/10A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Unidirectional extender | WJZL01A(B) | Sc/AL/T | L50～L300，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Hole slot needle base | WJZL05 A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Porous connecting piece | WJZL13A(B) | Sc/AL/T | L20～L200，interval1 | 2 |
|  | Arc connecting piece | WJZL14A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Track connecting piece | WJZL16A(B) | Sc/AL/T | L20～L100，interval2 | 1 |
|  | Threaded column | WJZL18A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Pitted nut | WJZL20A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | Hexagonal column | WJZL21A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 40 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 40 |
|  | Indenter | WJZL28A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval2 | 1 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval2 | 1 |
|  | earring | WJZL30A(B) | Sc/AL/T | L10～L100，interval2 | 3 |
|  | Adjustable universal hinge | WJZL32A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Connecting piece | WJZL36A(B) | Sc/AL/T | L40～L200，interval2 | 1 |
|  | Ball bowl gasket | WJZL40A(B) | Sc/AL/T | L5～L400，interval2 | 1 |

|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
| WJZⅢ-25A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 2 |
|  | 3-lug 2/3-ring | WJZH02A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 2 |
|  | Four lug half ring | WJZH03A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Standard Taylor rod | WJZW01A(B) | Sc/AL/T | L50～260，interval2 | 6 |
|  | Quick mount Taylor bar | WJZW02A(B) | Sc/AL/T | L50～260，interval2 | 4 |
|  | Diagonal brace | WJZL03A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Offset slot needle base | WJZL07A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Universal joint | WJZL08A(B) | Sc/AL/T | L5～L400，interval2 | 16 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | screw thread Unidirectional joint | WJZL10A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | Bolt column | WJZL17A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Quadrangular column | WJZL19A(B) | Sc/AL/T | L10～L150，interval2 | 2 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | Ring rod bolt | WJZL23A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 15 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 42 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 42 |
|  | earring | WJZL30A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Ring needle connecting rod | WJZL39A(B) | Sc/AL/T | L10～L300，interval2 | 1 |
|  | Marking card | WJZL41A(B) | AL/T | L5-L400，interval2 | 8 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |

| WJZⅢ-26A(B) | Six lug full ring | WJZH01A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | General ring | WJZH06A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 2/3 ring | WJZH07A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/3 ring | WJZH08A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Ordinary 1/2 ring | WJZH11A(B) | Sc/AL/Cf/PEEK/T | R40-R200，interval2 | 4 |
|  | Ordinary U-shaped foot ring | WJZH09/10A(B) | Sc/AL/Cf/PEEK/T | R40～R200，interval2 | 1 |
|  | Unidirectional extender | WJZL01A(B) | Sc/AL/T | L50～L300，interval2 | 1 |
|  | Bending bar | WJZL02A(B) | Sc/AL/T | L50～L350，interval2 | 1 |
|  | Threaded rod | WJZL04A(B) | Sc/AL/T | L50～L350，interval2 | 11 |
|  | Hole slot needle base | WJZL05A(B) | Sc/AL/T | L10～L100，interval2 | 4 |
|  | Half needle base | WJZL06A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Unidirectional joint | WJZL09A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | 90 ° conversion block | WJZL11A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | 90 ° conversion plate | WJZL12A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | 90 ° connecting piece | WJZL15A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Pitted bolt | WJZL20A(B) | Sc/AL/T | L5～L400，interval2 | 2 |
|  | bushing | WJZL22A(B) | AL/T | L5～L400，interval2 | 6 |
|  | bolt | WJZL24A(B) | Sc/AL/T | L10～L100，interval2 | 20 |
|  | Washer | WJZL25A(B) | Sc/AL/T | L5～L20,interval1 | 53 |
|  | Nut | WJZL26A(B) | Sc/AL/T | L5～L400，interval2 | 53 |
|  | spring | WJZL27A(B) | Sc/T | L15～L100，interval2 | 1 |
|  | Indenter | WJZL28A(B) | Sc/AL/T | L5～L400，interval2 | 1 |
|  | control | WJZL29A(B) | Sc/AL/T | L15～L100，interval2 | 1 |
|  | Quadrangular curved column | WJZL31A(B) | Sc/AL/T | L10～L150，interval2 | 1 |
|  | Adjustable universal hinge | WJZL32A(B) | Sc/AL/T | L10～L200，interval2 | 1 |

|  | Conversion chip | WJZL33A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| --- | --- | --- | --- | --- | --- |
|  | Single hole connecting piece | WJZL34A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Kirschner wire guide block | WJZL35A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Steering needle base | WJZL37A(B) | Sc/AL/T | L10～L200，interval2 | 1 |
|  | Universal rail connecting piece | WJZL38A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
|  | Connecting sleeve | WJZL42A(B) | Sc/AL/T | L10～L100，interval2 | 1 |
| 注：material quality Sc- Stainless steel，AL- aluminium alloy，Cf- carbon fibre，PEEK—Polyetheretherketone， T- titanium alloy |  |  |  |  |  |